---------------------------------------------------------------------------

---------------------------------------------------------------------------

BES Raw Data

A Guide to BES Data Record Formats

Version 1.0,      March 17, 1990

WRZ  and  CCZ

Type 33,48,49 are added.                           17-Oct-1990

---------------------------------------------------------------------------

---------------------------------------------------------------------------

This document describes the BES data record formats  produced by

BES on-line data acquisition system.

1.  BES data types

1.1  types of BES data record

Each BES data record in the BES on-line system has it's specified

type which are shown below.

--------------------------------------------------------------

Type  |  Length     |              Description

|  in LW      |

---------|-------------|--------------------------------------

1   |  Varible    | Event data from BES main trigger

|             |

|             |  subtype    subsystem

|             |     1          TOF

|             |     2         CDC-T

|             |     3         CDC-dE/dx

|             |     4         BES-TRG

|             |     5         MDC-T

|             |     6         MDC-dE/dx

|             |     8          BSC

|             |     9          ESC

|             |    10          MUC

---------|-------------|---------------------------------------

3   |  10+309     | BEPC monitor data

|             | For more information see BEPC_DATA.DOC

---------|-------------|---------------------------------------

4   |  10+2+336   | TOF Lasser test data

---------|-------------|---------------------------------------

7   |  10+40      | Luminosity monitor data(scl) from LUM-

|             | monitor at small angle

---------|-------------|---------------------------------------

8   |    7934     | online database

---------|-------------|---------------------------------------

11   |     11      | BegSetup - Begin run setup

---------|-------------|---------------------------------------

12   |     19      | Begin    - Begin run

---------|-------------|---------------------------------------

13   |     18      | Start    - Start data logging

---------|-------------|---------------------------------------

14   |     18      | Stop     - Stop  data logging

---------|-------------|---------------------------------------

15   |     11      | EndSetup - End   run setup

---------|-------------|---------------------------------------

16   |     18      | End      - End   run

---------|-------------|---------------------------------------

18   |(4+1+2)*4+3  | Scalers from BES Main Trigger

---------|-------------|---------------------------------------

33   | 33(132)*40  | summary file including 40 rec (132 byte

|             | per rec. see : [run.log]runstat.dat )

---------|-------------|---------------------------------------

48   |  10+1820    | initial paremeters for high voltage

|             | system

---------|-------------|---------------------------------------

49   | 10+7+326*2  | periodic monitoring high voltage data

---------|-------------|---------------------------------------

62   |  10+4560    | parameters for calculating Z in MUC

|             | (see : [run.db]mu3pnt.dat )

---------|-------------|---------------------------------------

63   |  10+7000    | Trigger file record

---------|-------------|---------------------------------------

101   |  10+(2+1)+24| Luminosity event data from LUM-monitor

|             | at small angle

|             |

|             |  subtype        subsystem

|             |    11         LUM-small angle

|             |

|             | (see : [bes.hshelp]evt_camac_frmt.doc)

---------|-------------|---------------------------------------

130-157 | 10 + fix_len| on_line calibration parameter records

---------|-------------|---------------------------------------

1.2  Record label

Each logical record begins with a 10 word block, defined as record

label, as follows.

_____________

1   |  Length   |   Inclusive # of longwords in record

2   |  Type     |   Data type of the record

3   |  Run      |   Run #

4   |  Rec      |   Record #

5   |  Time     |   Time in 30 msec units from Jan 1 00:00

6   |  chksum   |   A checksum for the logical record

7   |  Nstop    |   Bits indicating reason for a run stop

8   |  -        |   ( see : the note below )

9   |  -        |   ( see : the note below )

10   |  'BES'    |   A text string

-------------

Note that for records read out from CAMAC system words 8 & 9

give information regarding CAMAC errors.

2.  Data Record Formats

A detailed description of all record formats used in BES on-line

system are given in this section.

2.1  Type 1 : BES event record

BES event record contains event data coming from BES Main trigger

system.   Each record consists of a variable number of varible length

blocks ( as shown below ).   A pointer block, following 10 word label

at the start of the record,   contains a count  of the varible blocks

and a list of packed pointer/subtype flags. Each entry in the pointer

table is  a packed longword  containg the location of a subtype block

( longword location ip in IRAW(ip) )   and the subtype of that block.

Note the order of the pointers is thus unspecified.

Longword    content        comment

_______________

1-10   | 10 LW label |  Head word

---------------                              --

11   |     NPTRS   |  # of pointers to follow      |

12   |  S1  |  P1  |  Subtypes/Pointers Packed     |  Pointer

13   |  S2  |  P2  |  ...                          |  block

14   |  S3  |  P3  |  ...                          |

..   |  ... |  ... |  ...                          |

---------------                              --

The pointer is  an  absolute longword index from the beginning of

the event record to the 1st word of a subtype block.   In this word

there is the inclusive byte length for that subtype block.

The detailed description of data formats  for individual subtypes

can be found in the following files.

disk1:[bes.hshelp]evt_camac_frmt.doc

disk1:[bes.hshelp]evt_anal_frmt.doc

2.1.1 Subtype 1 : TOF data

The TOF data block consists of a byte-count ( 1 LW ) and a fixed

number of packed words ( 336 LW ).

Longword    content          comment

_________________

p1+ 0  |        |  BC  |  Byte Count

-----------------

1  | ID=1   | data |  packed word with channel ID & data

2  | ID=2   | data |  ...

3  | ID=3   | data |

...  |  ...   |  ... |  ...

336  | ID=336 | data |

-----------------

2.1.2 Subtype 2,3 : CDC TDC & dE/dx data

The CDC data block (both TDC & dE/dx) consists of a byte-count

( 1 LW ) and a variable number of packed words.

Longword    content          comment

_________________

p1+ 0  |        |  BC  |  Byte Count

-----------------

1  | ID1    | dat1 |  packed word with ID1 & data

2  | ID2    | dat2 |  ...              ID2 & data

3  | ID3    | dat3 |

...  |  ...   |  ... |  ...

-----------------

Note that the IDs for <CAMAC> and <Analysis> data formats are

compeletely different.

2.1.3 Subtype 4 : BES Main Trigger data

The data block for BES main trigger subsystem consists of a byte-

count ( 1 LW ) and a fixed number of packed words.

Longword    content          comment

__________________

p1+ 0  |        |  BC   |  Byte Count

|--------|-------|

1  |        | TR37  |  TR37 data from MDC Trk 2

2  |        | TR37  |  TR37 data from MDC Trk 4

3  |        | TR37  |  TR37 data from MDC Trk 6

|--------|-------|

4  |        | TR36  |  # of tracks

|--------|-------|

5  |        |   I-1 |  8 trg conditions for LVL-1

6  |        |   I-2 |  8 event types    for LVL-1

7  |        |  II-1 |  8 trg conditions for LVL-2

8  |        |  II-2 |  8 event types    for LVL-2

9  |        | III-1 |  8 trg conditions for LVL-3

10  |        | III-2 |  8 event types    for LVL-3

|--------|-------|

11  |        | TR16  |  BSC & ESC energy cut

------------------

12  |        | TR22  |  TOF Trg map for E_TOF

13  |        | TR22  |  TOF Trg map for E_TOF

14  |        | TR22  |  TOF Trg map for E_TOF

15  |        | TR21  |  TOF Trg map for B_TOF

16  |        | TR21  |  TOF Trg map for B_TOF

17  |        | TR21  |  TOF Trg map for B_TOF

|--------|-------|

2.1.4 Subtype 5,6 : MDC TDC & dE/dx data

The format of MDC data block (both TDC & dE/dx) is the same as for

CDC's. However the contents of ID words, which depends on configura-

tions of each subsystem,for CDC and MDC should be changed.

2.1.5 Subtype 8 : BSC Q data

See the words written for Subtype 5,6.

2.1.6 Subtype 9 : ESC Q data

See the words written for Subtype 5,6.

2.1.7 Subtype 10 : MUC Q data

See the words written for Subtype 5,6.

2.2  Type 4 : TOF Lasser data

TOF lasser data record contains event data coming from TOF Lasser

trigger system.

The format of this type of records is the same as BES main trigger

event data (i.e. type 1 )  except that  only  TOF data ( subtype 1 )

are  kept in each record.   And note that  the readout from reserved

BTQ & BTH  in Lassre case are signaled by the radiative source Ar.

2.3  Type 7 : Luminosity Monitor data

In the record of type 7, there are 3 scalers readout ( 12 channels

per scaler, totally 36 longwords ) followed  by three time durations

in the unit of msec ( one per scaler ).

Longword      content               comment

___________________

| 10 label words  |  Head

|--------|--------|

p1+ 0  |        |  BC    |  Byte Count

|--------|--------|

1  |        | SCL1-1 |  count for P1

2  |        | SCL1-2 |  count for P2

3  |        | SCL1-3 |  count for P3

4  |        | SCL1-4 |  count for P4

5  |        | SCL1-5 |  count for C1

6  |        | SCL1-6 |  count for C2

7  |        | SCL1-7 |  count for C3

8  |        | SCL1-8 |  count for C4

9  |        | SCL1-9 |  count for S1

10  |        | SCL1-10|  count for S2

11  |        | SCL1-11|  count for S3

12  |        | SCL1-12|  count for S4

|--------|--------|

13  |        | SCL2-1 |  count for P1S1

14  |        | SCL2-2 |  count for P2S2

15  |        | SCL2-3 |  count for P3S3

16  |        | SCL2-4 |  count for P4S4

17  |        | SCL2-5 |  count for C1S1

18  |        | SCL2-6 |  count for C2S2

19  |        | SCL2-7 |  count for C3S3

20  |        | SCL2-8 |  count for C4S4

21  |        | SCL2-9 |  unused

22  |        | SCL2-10|  unused

23  |        | SCL2-11|  unused

24  |        | SCL2-12|  unused

|--------|--------|

25  |        | SCL3-1 |  count for P1S1C3S3

26  |        | SCL3-2 |  count for P2S2C4S4

27  |        | SCL3-3 |  count for P3S3C1S1

28  |        | SCL3-4 |  count for P4S4C2S2

29  |        | SCL3-5 |  count for P1S1C3S3 delayed

30  |        | SCL3-6 |  count for P2S2C4S4 delayed

31  |        | SCL3-7 |  count for P3S3C1S1 delayed

32  |        | SCL3-8 |  count for P4S4C2S2 delayed

33  |        | SCL3-9 |  Random Sample

34  |        | SCL3-10|  Dead Time

35  |        | SCL3-11|  8 coincidences(13)

36  |        | SCL3-12|  8 coincidences(24)

|-----------------|

37  |          dT-1   |  delta T for scl1  in seconds

38  |          dT-2   |  delta T for scl2  in seconds

39  |          dT-3   |  delta T for scl3  in seconds

-------------------

Note :  The type of word 37,38,39 are  Real*4, while others are I*4.

2.4  Type 8 : On-line Database

This on-line record contains the BES database for current run,

which consists of two records.

the 1st record (7934 longwords):

Longword      content               comment

___________________

1-10    | 10 label words  |  Head

|-----------------|

11-7934 | BES_DB (part 1) |

-------------------

the 2nd record (2826 longwords):

Longword      content               comment

___________________

1-6     | 6  label words  |  Head

|-----------------|

7-2826  | BES_DB (part 2) |

-------------------

Note :  The type of head words are INTEGER*4 longwords, while others

are ASCII code text.

2.5  Type 11 : BegSetup

This record signals a data run to be begun. A 'BEGIN' run record

should normally follow.

Longword      content               comment

___________________

1-10  | 10 label words  |  Head

|--------|--------|

11  |      Run        |  Run number

-------------------

2.6  Type 12 : BEGIN

Longword      content               comment

___________________

1-10  | 10 label words  |  Head

|--------|--------|

11  |      '  MM'     |  Month   (01-12) in character format

12  |      '  DD'     |  Date    (01-31)

13  |      'YYYY'     |  Year    (e.g. 1990)

14  |      '  HH'     |  Hour    (00-24)

15  |      '  MM'     |  Minites (00-59)

16  |      '  SS'     |  Seconds (00-59)

17  |      ' mmm'     |  Msec    (0-999)

18  |      'BEGI'     |  Text    'BEGIN'

19  |      'N   '     |

-------------------

2.7  Type 13 : Start

Signals the start of data taking.

Longword      content               comment

___________________

1-10  | 10 label words  |  Head

|--------|--------|

11  |      '  MM'     |  Month   (01-12) in character format

12  |      '  DD'     |  Date    (01-31)

13  |      'YYYY'     |  Year    (e.g. 1990)

14  |      '  HH'     |  Hour    (00-24)

15  |      '  MM'     |  Minites (00-59)

16  |      '  SS'     |  Seconds (00-59)

17  |      ' mmm'     |  Msec    (0-999)

18  |      'GO  '     |  Text    'GO'

-------------------

2.8  Type 14 : Stop

Signals a pause in data taking, may be followed by end of run or

another Start record.

Longword      content               comment

___________________

1-10  | 10 label words  |  Head

|--------|--------|

11  |      '  MM'     |  Month   (01-12) in character format

12  |      '  DD'     |  Date    (01-31)

13  |      'YYYY'     |  Year    (e.g. 1990)

14  |      '  HH'     |  Hour    (00-24)

15  |      '  MM'     |  Minites (00-59)

16  |      '  SS'     |  Seconds (00-59)

17  |      ' mmm'     |  Msec    (0-999)

18  |      'STOP'     |  Text    'STOP'

-------------------

2.9  Type 15 : EndSetup

Signals an End of run follows.

Longword      content               comment

___________________

1-10  | 10 label words  |  Head

|--------|--------|

11  |      Run        |  Run number

-------------------

2.10  Type 16 : EndRun

Under normal circumstances this should be the last record of a run.

Longword      content               comment

___________________

1-10  | 10 label words  |  Head

|--------|--------|

11  |      '  MM'     |  Month   (01-12) in character format

12  |      '  DD'     |  Date    (01-31)

13  |      'YYYY'     |  Year    (e.g. 1990)

14  |      '  HH'     |  Hour    (00-24)

15  |      '  MM'     |  Minites (00-59)

16  |      '  SS'     |  Seconds (00-59)

17  |      ' mmm'     |  Msec    (0-999)

18  |      'END '     |  Text    'END'

-------------------

2.11  Type 18 : Scalers for BES Main Trigger system

In the record of type 18, there are 3 types of trigger scalers

(4,1,2 scalers/type & 4 channels/scaler, totally  28 longwords )

followed by 3 LWs containing time durations in the unit of sec

( one per type of scaler ).

The definition of scaler names for trigger system can be found

in disk file of < disk1:[besprod.src]TRIGRATE_NAMES.DEF >

Longword      content               comment

___________________

| 10 label words  |  Head

|--------|--------|

p1+ 0  |        |  BC    |  Byte Count = ( (4+1+2)*4 + 3 ) *4

|--------|--------|

1  |        | SCL1-1 |  Live

2  |        | SCL1-2 |  Pass LVL1

3  |        | SCL1-3 |  Dead

4  |        | SCL1-4 |  Good

|--------|--------|

5  |        | SCL2-1 |  Radial

6  |        | SCL2-2 |  Balance

7  |        | SCL2-3 |  Etotl

8  |        | SCL2-4 |  Etoth

|--------|--------|

9  |        | SCL3-1 |  Mu OR

10  |        | SCL3-2 |  CDC OR

11  |        | SCL3-3 |  Trk 1

12  |        | SCL3-4 |  Trk 2

|--------|--------|

13  |        | SCL4-1 |  Trg 1

14  |        | SCL4-2 |  Trg 2

15  |        | SCL4-3 |  Trg 3

16  |        | SCL4-4 |  Trg 4

|--------|--------|

17  |        | SCL5-1 |  Ntbb

18  |        | SCL5-2 |  Ntof1

19  |        | SCL5-3 |  Ntof2

20  |        | SCL5-4 |  unused

|-----------------|

21  |        | SCL6-1 |  CDC 3-1

22  |        | SCL6-2 |  CDC 3-2

23  |        | SCL6-3 |  CDC 3-3

24  |        | SCL6-4 |  CDC 3-4

|-----------------|

25  |        | SCL7-1 |  CDC 4-1

26  |        | SCL7-2 |  CDC 4-2

27  |        | SCL7-3 |  CDC 4-3

28  |        | SCL7-4 |  CDC 4-4

|-----------------|

29  |          dT-1   |  delta T for scl1-4  in seconds

30  |          dT-2   |  delta T for scl5    in seconds

31  |          dT-3   |  delta T for scl6-7  in seconds

-------------------

Note :  The type of word 29,30,31 are  Real*4, while others are I*4.

2.12  Type 33 : end run summary file

This record contains summary ascii file created at end run, that have

40 records (132 byte per record) in total.

Longword      content         comment

___________________

1-10   | 10 label words  |  Head

|-----------------|

11-43   | 33 LW of ASC    |  33 LW/record = 132 bytes/record

44-76   | 33 LW of ASC    |  (Totaly 40 records, i.e.

.     |       .         |          40 lines of readable ASC)

.     |       .         |

-1330 |-----------------|

2.13  Type 48 : initial parameters for high voltage system, logging

onto tape at every run beginning.

Longword      content         comment

___________________

1-10   | 10 label words  |  Head, INTEGER*4 long words

|-----------------|

11-1830 |_________________|  1820 LW for INTEGER*2 HV_3D(7,40,13)

NOTE:

(1). HV_3D is from disk files in HVDAT:CRATEsys_no.par_nam

(sys_no: in hex 1,2,...,9,A,B,C,D, par_nam: see below)

(2). The 3 dimensions of HV_3D(7,40,13) are:

7 params : VSAF, VNML, ANML, AMAX, DELV, DELV_DOWN, TRIP

40 chans : not fully used for some crates

13 crates: their sys_names and chans used are as following:

(3).  MD1 BS2 BS3 ES4 ES5 BT6 BT7 BT8 ET9 ET10 LUM11 CD12 MU13

ch_fr  1   1   1   1  29   1   1   1   1    1     1   24    1

ch_to 23  40  32  40  32  32  32  32  32   16    12   28   26

Used CH_TOT=326

2.14  Type 49 : High voltage  monitoring data, logging onto tape

periodically.

Longword      content         comment

___________________

1-10   | 10 label words  |  Head, INTEGER*4 long words

|-----------------|

11-17   | 7 long words    |  Time information in ASC

|-----------------|

18-665  | 2*326 long words|  INT*4 V_true(326),A_true(326)

|-----------------|

NOTE:

(1).  326 is just for CH_TOT=326 channels (see Type 48).

(2).  V_true is true voltage value for ALL CRATES.

A_true is true current value for ITALIA_CRATES(1-5,12-13),

and is param (VNML) value for LECROY_CRATES(6-11).

(3).      long word format:                taken by mask:

value(V_tr/A_tr)  on the  0-15th bits     '0000FFFF'x

status            on the 16-18th bits     '00070000'x

type(V_tr/A_tr)   on the    19th bit      '00080000'x

crate_No          on the 20-23th bits     '00F00000'x

channel_No        on the 24-31th bits     'FF000000'x

(4).      Some value's meaning:

type:      =1 for V_true

=0 for A_true

status:    =1 for OFF

=2 for TRIP

=4 for ON

2.15  Type 62 : three points correction parameters for Mu

This record contains parameters for calculating Z in MUC.

Longword      content         comment

___________________

1-10   | 10 label words  |  Head

|-----------------|

11-1530 | param 1         |

| chan  1-1520    |

|-----------------|

1531-3050 | param 2         |

| chan  1-1520    |

|-----------------|

3051-4570 | param 3         |

| chan  1-1520    |

|-----------------|

Note :  The type of 10 wds header are INTEGER*4 longwords, while others

for the parameter are REAL*4.

2.16  Type 63 : BES trigger file record

BES trigger file record contains the files describing BES trigger

system status for current run, which consists of:

file 1: BES_TRG:TR17.DAT   (energy threshold setting for TR17),

file 2: BES_TRG:TMMK.DAT   (trigger time marker table),

file 3: BES_TRG:TRGTBL.TMP (trigger condition table)'

file 4: BES_TRG:OVRD.DAT   (overriding for MDC track_finding),

file 5: BES_TRG:LOADM.DAT  (ROM loading for MDC track_finding).

Longword      content               comment

___________________

1-10   | 10 label words  |  Head

|-----------------|

11-410  | file 1, 400 wds |

|-----------------|

411-1010 | file 2, 600 wds |

|-----------------|

1011-1610 | file 3, 600 wds |

|-----------------|

1611-1810 | file 4, 200 wds |

|-----------------|

1811-7010 | file 5, 5200 wds|

-------------------

Note :  The type of 10 wds header are INTEGER*4 longwords, while other

7000 wds for files are ASCII code text.

2.17  Type 130-157 : on_line calibration parameter records

These records contain BES on_line calibration parameters for current

run.  However it doesn't tell online calibration correcting is actually

done or not, which depends on the setting of BES_DB for this run.

The record forms are more or less the same as following table, while

their contents and length are listed in the next table.

Longword      content         comment

___________________

1-10   | 10 label words  |  Head

|-----------------|

11    | ALPHA  | BETA   |\

|--------+--------|  for channel 1

12    | DELTA  | THRESH |/

|-----------------|

13    | ALPHA  | BETA   |\

|--------+--------|  for channel 2

14    | DELTA  | THRESH |/

|    .   |   .    |       .

|    .   |   .    |       .

|    .   |   .    |       .

-------------------

Note :  The type of 10 wds header are INTEGER*4 longwords, while others

for the parameter are INTEGER*2.

------------------------------------------------------------------------

| Type | SYS_BADC    | LWDs(HDs+DATA) | PARAs  | CHANs  |   Comment    |

+------+-------------+----------------+--------+--------+--------------+

| 130  | TOF_BADC 1  | 246 = 10 + 236 | *2=472 | /4=118 |  The TFQ_PED |

+------+-------------+----------------+--------+--------+ is from file |

| 131  | TOF_BADC 2  | 228 = 10 + 218 | *2=436 | /4=109 | PRE_TOF1.DAT |

+------+-------------+----------------+--------+--------+ ...,  before |

| 132  | TOF_BADC 3  | 228 = 10 + 218 | *2=436 | /4=109 | PEDACQ/UPD.  |

+------+-------------+----------------+--------+--------+--------------+

| 133  | TOF_BADC 1  | 246 = 10 + 236 | *2=472 | /4=118 |  The TFQ_PED |

+------+-------------+----------------+--------+--------+ is from file |

| 134  | TOF_BADC 2  | 228 = 10 + 218 | *2=436 | /4=109 |   TOF1.DAT   |

+------+-------------+----------------+--------+--------+   ..., after |

| 135  | TOF_BADC 3  | 228 = 10 + 218 | *2=436 | /4=109 | PEDACQ/UPD.  |

+------+-------------+----------------+--------+--------+--------------+

| 136  | CDC_T_BADC  | 778 = 10 + 768 | *2=1536| /4=384 |

+------+-------------+----------------+--------+--------+

| 137  | CDC_E_BADC  | 778 = 10 + 768 | *2=1536| /4=384 |

+------+-------------+----------------+--------+--------+

| 138  | MDC_T_BADC1 |1162 = 10 + 1152| *2=2304| /4=576 |

|  .   |     .       |       .        |   .    |   .    |

|      |             |                |        |        |

| 142  | MDC_T_BADC5 |1162 = 10 + 1152| *2=2304| /4=576 |

+------+-------------+----------------+--------+--------+

| 143  | MDC_E_BADC1 |1162 = 10 + 1152| *2=2304| /4=576 |

|  .   |     .       |       .        |   .    |   .    |

|      |             |                |        |        |

| 147  | MDC_E_BADC5 |1162 = 10 + 1152| *2=2304| /4=576 |

+------+-------------+----------------+--------+--------+

| 148  | BSC_BADC 1  |3370 = 10 + 3360| *2=6720| /4=1680|

|  .   |     .       |       .        |   .    |   .    |

|      |             |                |        |        |

| 151  | BSC_BADC 4  |3370 = 10 + 3360| *2=6720| /4=1680|

+------+-------------+----------------+--------+--------+

| 152  | ESC_BADC 1  |2770 = 10 + 2760| *2=5520| /4=1380|

|  .   |     .       |       .        |   .    |   .    |

| 155  | ESC_BADC 1  |2770 = 10 + 2760| *2=5520| /4=1380|

+------+-------------+----------------+--------+--------+

| 156  | MU_BADC 1   |1866 = 10 + 1856| *2=3712| /4=928 |

| 157  | MU_BADC 2   |1194 = 10 + 1184| *2=2368| /4=592 |

+------+-------------+----------------+--------+--------+

2.18  Type 101 : Luminosity event record

Luminosity event record contains event data coming from luminosity

monitor trigger system.

The record format is same as for BES event record (See section 2.1)

except that the former only contains data block of subtype 11.   Note

also that  one longword included  in the pointer block, which follows

the packed longword, always keep zero and is actually useless.

2.18.1 Subtype 11 : Luminosity monitor data

The data block for BES luminosity monitor consists of a byte-count

( 1 LW ) and a fixed number of packed words ( 23 LW ).

Longword    content          comment

___________________

p1+ 0  |        |  BC    |  Byte Count

|--------|--------|

1  |        | ADC(1) |  ADC for P counter 1

2  |        | ADC(2) |  ADC for P counter 2

3  |        | ADC(3) |  ADC for P counter 3

4  |        | ADC(4) |  ADC for P counter 4

|--------|--------|

5  |        | ADC(5) |  ADC for C counter 1

6  |        | ADC(6) |  ADC for C counter 2

7  |        | ADC(7) |  ADC for C counter 3

8  |        | ADC(8) |  ADC for C counter 4

|--------|--------|

9  |        | ADC(9) |  ADC for S counter 1

10  |        | ADC(10)|  ADC for S counter 2

11  |        | ADC(11)|  ADC for S counter 3

12  |        | ADC(12)|  ADC for S counter 4

|--------|--------|

13  |        | TDC(1) |  TDC for P counter 1

14  |        | TDC(2) |  TDC for P counter 2

15  |        | TDC(3) |  TDC for P counter 3

16  |        | TDC(4) |  TDC for P counter 4

|--------|--------|

17  |        | TDC(5) |  TDC for S counter 1

18  |        | TDC(6) |  TDC for S counter 2

19  |        | TDC(7) |  TDC for S counter 3

20  |        | TDC(8) |  TDC for S counter 4

|--------|--------|

21  |        | Reg(1) |  chan 1 in Output register

22  |        | Reg(2) |  chan 2 in Output register

23  |        | Reg(3) |  chan 3 in Output register

------------------