STATUS OF BEPC’98 OPERATION

BEPC Group, presented by Y. Luo

IHEP, P. O. Box 918-9, 100039, Beijing, P. R. China

Abstract

1998 is the tenth year of Beijing Electron-Positron Collider (BEPC) since its first operation in 1988. This time, BEPC’s operation was started on March 10, and will be finished in July. In this paper, the status of BEPC’98 operation is reviewed, which includes the following parts: the general description, start-up for BEPC/BES, performance for R scan, machine study, performance for synchrotron radiation and, at last, conclusion.

(1) General Description

BEPC’98 operation was started on March 10 and will be finished in July. Table 1 shows the schedule of this time's operation.

Table 1 The schedule of BEPC’98 operation

Period                                 Task

March 10 ~    March 25   Start-up for BEPC

March 25 ~    April  6      Start-up for BES

April 7    ~    April 16      Machine Study(MD)

April 17   ~    May 26       R Scan

May 26   ~     June  5        Machine Study

June 6     ~                        Synchrotron Radiation

It can be seen in Table 1 that the start-up for BEPC was from March 10 to March 25 and the start-up for BES was from March 25 to April 6. From April 7, we began to perform for R scan and machine study. The operation for synchrotron radiation was started on June 6, which will last for about one month. Figure 1 displays the statistics of BEPC’98 operation.

Fig. 1 The statistics of BEPC’98 operation

From March 10 to June 1, the total operation time is about 1996hrs, among which the start-up for BEPC took up 15.7%, BES 37.9%, machine study 26.8%, injection 11.6%, and faults 8.0%.

(2) Start-up for BEPC/BES

The time occupied by the start-up for BEPC this time was shorter than that of BEPC 96/97operation [1]. In the first shift, the electron beam was injected into the ring without any corrector, and in the second shift of the same day, the electron beam was accumulated more than 70mA. On the fifth day, the collision at the J¤y energy (1.55GeV) was achieved. And during the start-up for BEPC, the new configurations for synchrotron radiation were successfully commissioned. All the above was owed to good preparations before BEPC'98 operation. Table 2 lists some work done before BEPC’98 operation. Start-up for BES was from March 25 to April 6.

Table 2 Some work done before BEPC’98 operation

· The linac was started in last Oct. Some components were repaired and replaced.

· The alignment for the ring was carried out from last September to November.

· Configurations were prepared and stimulated done by Accelerator Physics Group.

· The control system and beam instrumentation system were improved.

· The staffs taking part in the BEPC operation were trained.

· Reform of the operation regulation was taken.

(3) Performance for R-Scan

One of the main tasks in BEPC'98 operation is R-scan. This time we performed for R-scan from April 25 to May 26, at the center-of-mass energies of 3.2GeV, 3.4GeV, 3.55GeV, 4.6GeV, 5.0Gev and 2.6GeV, totally six energy points. In order to reduce the measurement error DR/R, the collaborative efforts of both BEPC and BES are needed. From machine side, the key point is to obtain the reasonable luminosity in such a wide energy range. After this run’s R-scan, some progresses have been achieved.

During R scan, all the BEPC staffs participating in the operation worked hard. It took quite short time to commission new configurations at different beam energies, usually several hours. Table 3 gives some typical machine parameters at different beam energies.

Table 3 The collider parameters at different beam energies

| E (GeV) | Ie++Ie- (mA) | L( ´1030 cm-2s-1) |  (hrs) | S/N |
| --- | --- | --- | --- | --- |
| 3.2 | 29.1 | 3.4 | 4.3 | 158 |
| 3.55 | 30.0 | 2.7 | 7.8 | 40 |
| 4.6 | 26.6 | 2.4 | 7.8 | 42 |
| 5.0 | 21.6 | 2.8 | 6.5 | 17 |
| 2.6 | 19.6 | 1.3 | 7.0 | 133 |

It can be seen from Table 3 that the beam current was not more than 30mA, which was required by BES. When the beam current was higher or the beam energy was higher, the ratio of signal to noise of BES was low. The noise was estimated partly from BES, and partly from BEPC [2]. From BEPC, only around the central mass energy 2.6GeV, the maximum beam current is limited by the beam-beam interaction. At other energy, the machine could provide higher beam currents. For example, at the center-of-mass energy 4.0GeV, the beam current reached 60mA in this round’s BEPC operation. How to reduce the noise is being considered.

One of the progresses obtained during this time's R-scan is that the maximum beam current at the center-of-mass energy 2.6GeV reached 22.0mA, and the maximum luminosity reached 1.26´1030 cm-2s-1 , almost twice of that of last year (0.68´1030 cm-2s-1 ).

(4) Machine Study

In BEPC’98 operation, we have spent much time and put many efforts on machine studies. Table 4 lists some items done. Among them, the single interaction point experiment and the pretzel scheme experiment are the most important, which both were aimed at obtaining higher luminosity with new schemes.

Table 4 Some experiments done in BEPC’98 operation

· Single interaction point experiment

· Multi-bunch scheme experiment

· Photoelectron instability study

· Beam-beam interaction

· Beam response matrix

· RF feedback experiment

· 2.0 GeV collision

· 1.55 GeV injection

· Luminosity vs. by*, and so on.

The photoelectron instability experiment was done jointly by IHEP, China and KEK, Japan. It was the final experiment of its series on BEPC. The experiment has indicated that the octupole can damp the photoelectron instability.

4.1 Single Interaction Point Experiment [3]

From accelerator physics, reducing the interaction points along the ring will increase the beam current. At present, BEPC is being run with two interaction points, and with only one detector BES at the south interaction point. Single interaction point experiment separated the electron and positron beams at the north interaction point with a pair of electro-static separators, so the total interaction points were reduced to one.

After hard work, some progresses were achieved. At the J¤y energy, with the βy=7cm, the colliding beam current reached åI= 42.7mA, and the maximum luminosity reached 4.4´1030  cm-2s-1, which is increased by a factor of 20%, compared to the maximum luminosity of two interaction points collision withβy=5cm in this time’s BEPC operation. Next step is to optimize the collision configuration with βy=5cm to increase the luminosity further.

4.2 Pretzel scheme experiment [4]

There is only one beam pipe in BEPC, in which the electron and positron beams are both contained. In order to realize multi-bunch collision, the electron and positron beams must be separated at all the parasitic interaction points. In BEPC II, the pretzel scheme will be adopted to realize multi-bunch collision. Pretzel scheme uses a pair of electro-static separators to create different vertical closed orbits for electron and positron beams, and separates the two beams at all the parasitic interaction points. Figure 2 displays the layout of the vertical pretzel scheme.

In our experiment, we used a pair of electro-static separators near the south interaction point to create the different vertical closed orbits for electron and positron beams. Of course, since we were still short of another pair of electro-static separators near the south interaction point for injection, multi-bunch collision couldn't be achieved.

Finally, with the vertical separation and horizontal injection, multi-bunch injection of electron beam was achieved, and 1´1 collision (one bunch electron and one bunch positron) at the J¤y energy with βy=5 cm was obtained.

Figure 2  Layout of vertical pretzel scheme  ( Qy=7.2 )

(5) Synchrotron Radiation Performance

Operation for synchrotron radiation was started from June 6, and will last for about one month. During the operation for synchrotron radiation, two wigglers (3w1 and 4w1) are used to increase the critical energy of the synchrotron radiation light. In this time’s performance, we planed to increase the beam current to 100 mA at 2.2 GeV. And we wish to increase the beam energy to 2.5 GeV by the end of this operation.

(6) Conclusion

With the collaboration of BEPC and BES in the round’s BEPC operation, we have succeeded in the performance for R-scan. Still we have made progresses in machine studies. The machine performs well for synchrotron radiation application now in progress.

Acknowledgments

The author would like to give his thanks to all the colleagues working on BEPC.

References

[1] C. Zhang, Proceedings of BES'97

[2] J. Li, these proceedings

[3] X. A. Luo et al., Proceedings of BEPC'98

[4] Q. Qin et al., Proceedings of BEPC’98